## Double relay watchdog user manual

## Features & Specifications

– Applicable to a variety of computer motherboards with a Type-A USB interface

– Applicable for windows x86/x64 system includes winxp, win7, win8, win10

– Includes driver compatible with a variety of motherboards

– Includes with Desktop software, It can set many functions with the program as your need. Better than no software version.

– Dual Relay system with Reboot timeout that are adjustable from 10 – 1270 seconds

- Specified Program Monitoring

- Scheduled Restart

- Website Monitoring

- Video Card Monitoring

- Run Log Query

- Support Forced Power off Restart

- Support Connect electricity Automatic Starting up

Installation steps

1.Disconnected watchdog,Click “WatchdogDriverSetup.exe”.

2.Disconnected cable,Insert the watchdog into any USB port,Until the drive is installed.

- 3.Click “PCWatchdog”,If normal, LED flashes 1 times in 1 second.

- 4.Here is double relay version connection diagram

Reset cable,Arbitrary direction

Reset cable,Arbitrary direction

Power on cable,Any direction

Reset cable,Arbitrary direction

- Here is another version of the red cyclone connection diagram